Precision Spraying Map Logic and End-to-End Practical Solution

Team 3 – Application | Issues #32 and #33 | Phase 4: Integrate

# 1. Introduction

Our system uses a computer vision model mounted on agricultural drones or fixed poles to classify crop disease and pest damage from leaf images, and then turns those results into a precision spraying map. Member 6 has already built, trained and exported the classifier (MobileNetV3-Large, PyTorch, exported as TorchScript). This document describes the system that sits on top of that classifier. It does not replace or retrain it.

The work is split into two parts. Part A is the design of the spraying map logic: how one classification becomes a severity score, how many scores become zones, how zones are chosen for spraying so that pesticide use is minimised, and how the farmer verifies the map before any action is taken. Part B is the practical solution: a Python pipeline that loads the exported model, runs a batch of images, attaches GPS and timestamps, applies the Part A logic and outputs a structured spraying map. The full code is in the labelled code block in the appendix.

Two project requirements shape every decision below. First, the solution must stay compatible with Python, PyTorch and OpenCV. Second, farmers must always be able to review the AI results before any spraying action is taken. The second requirement is built into the logic itself and is not an optional extra.

# 2. Part A – Precision Spraying Map Logic

## 2.1 Detection-level severity score

Each image gives one result: a class label and a confidence score. Member 6’s model has 12 classes, which include healthy leaves, diseases (such as leaf blight and verticillium wilt) and pest damage (such as fall armyworm). Healthy leaves must never create spraying pressure, and a serious problem must count for more than a minor one. For this reason the severity of one detection is calculated as:

severity = class weight × model confidence (%)

The result is a number from 0 to 100. The class weight (0 to 1) reflects how destructive and how fast-spreading the problem is. The confidence factor means that a shaky prediction counts for less than a confident one. For example, leaf blight at 88% confidence scores 0.70 × 88 = 61.6, while a healthy leaf at 95% confidence scores 0, whatever the confidence.

Table 1: Class weights (placeholder values to be confirmed by an agronomist)

| Class (farmer-facing label) | Weight |
| --- | --- |
| Healthy | 0.00 |
| Leaf spot, Leaf beetle | 0.40 |
| Grasshopper | 0.50 |
| Septoria leaf spot | 0.60 |
| Leaf blight, Leaf curl | 0.70 |
| Fall armyworm, Streak virus, Verticillium wilt | 0.90 |

Detections below Member 6’s 70% confidence threshold are still scored, but they are marked as low confidence. These marks are carried forward so that the zone-level verification step (section 2.4) can use them. The weights are our own assumptions. They are not taken from the dataset, so they must be validated with an agronomist before real use.

## 2.2 Aggregating detections into zones

One image covers only a small patch of leaf, so decisions cannot be made image by image. The field is divided into a regular grid of 20 m × 20 m zones. GPS coordinates are converted into metres from a field origin, and each detection is placed in the grid cell that contains it. Each zone is identified by its row and column (for example R2C1). A grid was chosen over clustering because it is simple to explain, produces stable and repeatable zone boundaries between flights, and is easy for the farmer to check on a map.

The severity of a zone combines the average and the worst detection in it:

zone severity = 0.7 × mean + 0.3 × max

The mean shows how widespread the problem is across the zone. The maximum stops one severe hotspot from being averaged away by many healthy leaves. As an example, a zone with five detections scoring 61.6, 36.0, 0, 0 and 0 has a mean of 19.5 and a maximum of 61.6, so its zone severity is 0.7 × 19.5 + 0.3 × 61.6 = 32.1. The mean alone would only have placed this zone in the monitor band, but the strong hotspot correctly pushes it over the spraying threshold.

Each zone also records its number of detections, its most common problem class, the share of low-confidence results, and the share of poor GPS fixes. These feed directly into the verification step.

## 2.3 Resource-optimisation rule

The goal is to use as little pesticide as possible, so only zones above a severity threshold are flagged for spraying and the rest of the field is left alone. The rule has three parts:

- Threshold: a zone is a spraying candidate only if its severity is 30 or higher. Zones between 15 and 30 are marked for monitoring (re-scan in two to three days). Zones below 15 need no action.

- Budget cap: candidates are ranked from worst to best, and no more than 50% of the zones can be funded for spraying in one run. Candidates beyond the cap are marked as deferred and re-scanned. This stops a bad scan or a mis-set threshold from recommending spraying of the whole field.

- Dose scaling: a zone with severity of 60 or higher is high priority and gets the full dose (100%). A zone from 30 to 60 is standard priority and gets a reduced dose (60%). The dose percentages are placeholders to be set with an agronomist against the product label.

Table 2: Action bands (all thresholds are starting points to be tuned on real data)

| Zone severity | Recommended action | Dose |
| --- | --- | --- |
| 60 and above | SPRAY – HIGH PRIORITY | 100% |
| 30 to below 60 | SPRAY – STANDARD | 60% |
| 15 to below 30 | MONITOR – re-scan | 0% |
| Below 15 | NO ACTION | 0% |

The pesticide saving is estimated as the share of zones that are not sprayed, compared with blanket-spraying the whole field: saving = (1 − sprayed zones ÷ total zones) × 100. This gives a direct figure for the business case. It is a zone-count estimate and does not account for dose scaling, so the real saving would be slightly higher.

## 2.4 Farmer verification and the risk register

The risk register contains an “incorrect spraying map” risk, which is mitigated by letting the farmer verify GPS placement and zone boundaries before any action is taken. This is built into the logic in three layers.

- Automatic flags. Every zone is checked for reasons the map might be wrong, and gets needs_verification = true if any apply: more than 30% of its GPS fixes are worse than 5 m (position and boundary doubtful); it contains fewer than 3 images (too little evidence); or more than 30% of its results are below the 70% confidence threshold (the classification itself is doubtful). The reasons are written out in plain text so the farmer knows what to check.

- Nothing is auto-approved. Every zone starts with the status AWAITING_FARMER_REVIEW. The output gives the zone centre in GPS coordinates and its boundaries, so the farmer can check them on a map.

- A release gate. The function release_for_spraying() is the only way a zone can enter the spray plan. A zone is released only if it has a spraying recommendation and the farmer has explicitly approved it. A zone without an explicit approval is held, even if it has no warning flags. We tested this directly: with no approvals supplied, zero zones were released.

The flags decide where the farmer should look hardest. They do not decide who approves. Even a clean zone needs the farmer’s approval, which satisfies the requirement that farmers can always review AI results before any spraying action.

# 3. Part B – Practical Solution Integration

## 3.1 Pipeline overview

The script spraying_pipeline.py runs the full journey from image to approved spray plan in the following stages:

- Load the model. The exported TorchScript file (leaf_classifier.pt) is loaded with torch.jit.load.

- Pre-process each image. The same steps as Member 6 are used: OpenCV validate, resize to 224 × 224, convert to RGB, then ImageNet normalisation. A corrupt image is skipped and reported, and never reaches the model.

- Classify. Softmax is applied to the model output to get the class and the confidence, as in Member 6’s predict_image().

- Attach GPS and timestamp to every result.

- Score, aggregate and optimise using the Part A logic (sections 2.1 to 2.3).

- Flag for verification and hold every zone until the farmer decides (section 2.4).

- Output the structured spraying map as JSON.

## 3.2 What is real and what is placeholder

As in Member 6’s report, the code marks each part as [REAL] or [PLACEHOLDER] in its comments.

Table 3: Real logic versus simulated parts

| Part | Status | Notes |
| --- | --- | --- |
| Severity scoring, zone aggregation, resource optimisation | Real | Final logic, thresholds still to be tuned |
| Verification flags and release gate | Real | Final logic |
| Image pre-processing and TorchScript inference | Real | Runs when leaf_classifier.pt and torch are present |
| Batch of 120 images | Placeholder | Random noise images created by the script |
| GPS coordinates, GPS accuracy, timestamps | Placeholder | Simulated; real values come from drone/pole metadata |
| Simulated classifier | Placeholder | Used only if the model file or torch is missing |
| Farmer approvals in the demo | Placeholder | Real system: farmer approves on the dashboard |

## 3.3 Output format

The final output is a list of zone entries. Each entry has the zone ID, severity, recommended action, recommended dose, a verification flag with its reasons, the dominant problem class, the number of detections, the centre GPS coordinates, the zone boundaries and a status. A summary block gives the totals. Table 4 shows the result of a run on the simulated field (a 60 m × 60 m field, 9 zones, 120 images). A disease hotspot was simulated in the north-east corner, a milder patch next to it, and poor GPS in the south-east corner to test the flags.

Table 4: Sample output on simulated data (shortened)

| Zone | Severity | Recommended action | Verify? | Status |
| --- | --- | --- | --- | --- |
| R0C0 | 10.7 | NO ACTION | No | Awaiting farmer review |
| R0C1 | 14.1 | NO ACTION | No | Awaiting farmer review |
| R0C2 | 12.7 | NO ACTION | Yes | Awaiting farmer review |
| R1C0 | 10.6 | NO ACTION | No | Awaiting farmer review |
| R1C1 | 10.1 | NO ACTION | No | Awaiting farmer review |
| R1C2 | 13.7 | NO ACTION | No | Awaiting farmer review |
| R2C0 | 14.8 | NO ACTION | No | Awaiting farmer review |
| R2C1 | 38.0 | SPRAY - STANDARD | No | Approved by farmer |
| R2C2 | 74.9 | SPRAY - HIGH PRIORITY | No | Approved by farmer |

In this run 2 of 9 zones were recommended for spraying, which is an estimated saving of 77.8% compared with blanket spraying. 1 zone (R0C2) was flagged because all of its GPS fixes were poor. The simulated farmer approved the two spraying zones. These numbers come from invented data and only show that the pipeline works. They are not evidence of real-world performance.

## 3.4 How to run

- Install the requirements: pip install numpy opencv-python torch.

- Place Member 6’s leaf_classifier.pt in the same folder as the script.

- Run python spraying_pipeline.py. The map is printed and saved as spraying_map.json.

If the model file or torch is missing, the script says so and continues with the simulated classifier, so the Part A logic can still be demonstrated.

## 3.5 Assumptions and limitations

- Testing status. The pipeline was run end to end in simulated mode, and the verification gate was tested directly. The real TorchScript path follows Member 6’s preprocessing but has not yet been run against the actual leaf_classifier.pt, because the model file and PyTorch were not available in the environment used for testing. It should be run once the file is available.

- Class weights, thresholds and doses are starting values and need agronomist input and tuning on real data.

- Pests are in the model. Three of the 12 classes are pests, not diseases, so the system treats them in the same severity framework. The weights may need to differ in practice.

- Flat-earth approximation. GPS is converted to metres with a simple approximation, which is accurate enough for a single farm but not for very large areas.

- Fixed grid. A fixed 20 m grid can split one infected patch across two zones. Smoothing or merging neighbouring zones is a possible future improvement.

- Latency on drone hardware has not been benchmarked, as noted in Member 6’s report.

# Appendix: Code Block

CODE BLOCK – spraying_pipeline.py (Part B: end-to-end pipeline, including the Part A logic)

"""

spraying_pipeline.py

====================

End-to-end pipeline: drone/pole image batch -> CNN classification ->

severity scoring -> zone aggregation -> resource optimization ->

farmer verification gate -> structured spraying map.

Sits on top of Member 6's exported TorchScript model (leaf_classifier.pt).

It does NOT retrain or replace the classifier.

Legend used in the comments below (same idea as Member 6's dummy-data flags):

[REAL]        = real logic that will be kept in the final system

[PLACEHOLDER] = simulated data/behaviour, replace when hardware exists

Run:  python spraying_pipeline.py

Needs: numpy, opencv-python  (+ torch if leaf_classifier.pt is available)

"""

import json

import math

import os

import random

import tempfile

from collections import Counter, defaultdict

from datetime import datetime, timedelta, timezone

import cv2

import numpy as np

# =============================================================================

# 0. CONFIGURATION  [REAL]  (all thresholds are starting points, to be tuned)

# =============================================================================

MODEL_PATH = "leaf_classifier.pt"      # Member 6's TorchScript export

IMG_SIZE = 224                         # must match Member 6's IMG_SIZE

IMAGENET_MEAN = np.array([0.485, 0.456, 0.406], dtype=np.float32)

IMAGENET_STD = np.array([0.229, 0.224, 0.225], dtype=np.float32)

CONFIDENCE_THRESHOLD = 70.0            # percent, same as Member 6's B3

# Must stay in sync with CLASS_NAMES in Member 6's train_deploy_demo.py

CLASS_NAMES = [

"Fall armyworm", "Grasshopper", "Healthy (Maize)", "Leaf beetle",

"Leaf blight (Maize)", "Leaf spot", "Streak virus",

"Healthy (Tomato)", "Leaf blight (Tomato)", "Leaf curl",

"Septoria leaf spot", "Verticillium wilt",

]

# Base severity weight per class (0 = harmless, 1 = most destructive).

# ASSUMPTION: these are placeholder expert guesses. An agronomist should

# confirm them before real use.

CLASS_SEVERITY_WEIGHT = {

"Healthy": 0.00,

"Leaf spot": 0.40,

"Leaf beetle": 0.40,

"Grasshopper": 0.50,

"Septoria leaf spot": 0.60,

"Leaf blight": 0.70,

"Leaf curl": 0.70,

"Fall armyworm": 0.90,

"Streak virus": 0.90,

"Verticillium wilt": 0.90,

}

CELL_SIZE_M = 20.0            # zone = 20 m x 20 m grid cell

MIN_DETECTIONS_PER_ZONE = 3   # fewer than this = weak evidence -> verify

MEAN_WEIGHT, MAX_WEIGHT = 0.7, 0.3   # zone severity blend (see aggregate)

SPRAY_THRESHOLD = 30.0        # zone severity >= this is a spray candidate

HIGH_THRESHOLD = 60.0         # zone severity >= this is high priority

MONITOR_THRESHOLD = 15.0      # between this and SPRAY_THRESHOLD = monitor

MAX_SPRAY_FRACTION = 0.50     # budget: never spray more than 50% of field

DOSE_HIGH_PCT, DOSE_STANDARD_PCT = 100, 60

GPS_ACCURACY_LIMIT_M = 5.0    # worse than this -> farmer must check position

LOW_CONF_ZONE_SHARE = 0.30    # >30% low-confidence detections -> verify

POOR_GPS_ZONE_SHARE = 0.30    # >30% poor GPS fixes in a zone -> verify

# =============================================================================

# 1. MODEL LOADING + INFERENCE

# =============================================================================

def display_label(class_name):

"""'Healthy (Maize)' -> 'Healthy' (same as Member 6's display_label)."""

return class_name.split(" (")[0]

def preprocess(image_path):

"""[REAL] Same steps as Member 6: validate -> resize -> RGB -> normalise.

Equivalent to their eval_transform (ToTensor + ImageNet Normalize)."""

img = cv2.imread(str(image_path))

if img is None:                       # "validate" step

raise ValueError(f"Could not read image: {image_path}")

img = cv2.resize(img, (IMG_SIZE, IMG_SIZE))

img = cv2.cvtColor(img, cv2.COLOR_BGR2RGB)

arr = img.astype(np.float32) / 255.0

arr = (arr - IMAGENET_MEAN) / IMAGENET_STD

return np.transpose(arr, (2, 0, 1))   # HWC -> CHW

class TorchScriptClassifier:

"""[REAL] Loads Member 6's exported model and classifies one image."""

def __init__(self, path):

import torch                      # imported here so the script still

self.torch = torch                # runs (in simulation) without torch

self.model = torch.jit.load(path, map_location="cpu")

self.model.eval()

def predict(self, image_path):

t = self.torch.from_numpy(preprocess(image_path)).unsqueeze(0)

with self.torch.no_grad():

probs = self.torch.softmax(self.model(t), dim=1)

conf, idx = self.torch.max(probs, dim=1)

return display_label(CLASS_NAMES[idx.item()]), conf.item() * 100

class SimulatedClassifier:

"""[PLACEHOLDER] Stand-in used only when leaf_classifier.pt or torch is

missing. Returns pre-decided (label, confidence) pairs so the demo has a

realistic spatial pattern. Delete once the real model is available."""

def __init__(self, truth):

self.truth = truth                # {image_path: (label, conf_pct)}

def predict(self, image_path):

preprocess(image_path)            # still exercises the validate step

return self.truth[image_path]

def load_classifier(sim_truth):

if os.path.exists(MODEL_PATH):

try:

clf = TorchScriptClassifier(MODEL_PATH)

print(f"[model] Loaded real TorchScript model: {MODEL_PATH}")

return clf

except ImportError:

print("[model] torch not installed -> using SIMULATED classifier")

else:

print(f"[model] {MODEL_PATH} not found -> using SIMULATED classifier")

return SimulatedClassifier(sim_truth)

# =============================================================================

# 2. PLACEHOLDER INPUTS: dummy image batch + simulated GPS/timestamps

# =============================================================================

def simulate_batch(folder, n_images=120, seed=7):

"""[PLACEHOLDER] Creates dummy images and fake GPS/timestamps.

In production, GPS + timestamp come from the drone/pole metadata.

The disease 'hotspot' in the north-east is invented for the demo."""

rng = random.Random(seed)

origin_lat, origin_lon = -26.6700, 27.9300       # simulated field corner

field_m = 60.0                                   # 60 m x 60 m field

start = datetime(2026, 10, 8, 6, 0, tzinfo=timezone.utc)

batch, truth = [], {}

for i in range(n_images):

x, y = rng.uniform(0, field_m), rng.uniform(0, field_m)   # metres

lat = origin_lat + y / 111_320.0

lon = origin_lon + x / (111_320.0 * math.cos(math.radians(origin_lat)))

in_hotspot = x > 40 and y > 40               # north-east corner

in_mild = 20 < x <= 40 and y > 40            # mild patch next to it

if in_hotspot:

label = rng.choice(["Fall armyworm", "Leaf blight", "Streak virus"])

conf = rng.uniform(75, 98)

elif in_mild:

label = rng.choice(["Leaf spot", "Leaf spot", "Leaf curl", "Healthy"])

conf = rng.uniform(70, 92)

else:

label = "Healthy" if rng.random() < 0.9 else "Leaf spot"

conf = rng.uniform(70, 99)

if rng.random() < 0.06:                      # occasional unsure result

conf = rng.uniform(40, 69)

path = os.path.join(folder, f"img_{i:03d}.jpg")

cv2.imwrite(path, np.random.randint(0, 255, (256, 256, 3), np.uint8))

truth[path] = (label, conf)

batch.append({

"image_path": path,

"latitude": lat,

"longitude": lon,

# fake GPS quality: good almost everywhere, poor in the south-east

# corner so the verification flag has something to catch

"gps_accuracy_m": 8.0 if (x > 40 and y < 20) else rng.choice([1.0, 1.5, 2.0, 3.0]),

"timestamp": (start + timedelta(seconds=10 * i)).isoformat(),

})

return batch, truth, (origin_lat, origin_lon)

# =============================================================================

# 3. PART A LOGIC  [REAL]

# =============================================================================

def severity_score(label, confidence_pct):

"""A1. One detection -> severity 0-100.

severity = class weight x model confidence. Healthy is always 0, and a

shaky prediction counts for less than a confident one."""

weight = CLASS_SEVERITY_WEIGHT.get(label, 0.5)   # unknown class: middle

return weight * confidence_pct               # (conf is already 0-100)

def to_local_metres(lat, lon, origin):

"""Converts GPS to metres east/north of the field origin (flat-earth

approximation, fine for a single farm)."""

o_lat, o_lon = origin

y = (lat - o_lat) * 111_320.0

x = (lon - o_lon) * 111_320.0 * math.cos(math.radians(o_lat))

return x, y

def aggregate_zones(detections, origin):

"""A2. Groups detections into grid zones and scores each zone.

zone severity = 0.7 x mean + 0.3 x max

- the mean shows how widespread the problem is in the zone

- the max stops one severe hotspot being averaged away by healthy leaves

"""

cells = defaultdict(list)

for d in detections:

x, y = to_local_metres(d["latitude"], d["longitude"], origin)

col, row = int(x // CELL_SIZE_M), int(y // CELL_SIZE_M)

cells[(row, col)].append(d)

zones = []

for (row, col), items in sorted(cells.items()):

sev = [d["severity"] for d in items]

zone_sev = MEAN_WEIGHT * (sum(sev) / len(sev)) + MAX_WEIGHT * max(sev)

diseased = [d["label"] for d in items if d["label"] != "Healthy"]

# zone centre converted back to GPS so the farmer can check placement

cx, cy = (col + 0.5) * CELL_SIZE_M, (row + 0.5) * CELL_SIZE_M

c_lat = origin[0] + cy / 111_320.0

c_lon = origin[1] + cx / (111_320.0 * math.cos(math.radians(origin[0])))

zones.append({

"zone_id": f"R{row}C{col}",

"row": row, "col": col,

"centre_latitude": round(c_lat, 6),

"centre_longitude": round(c_lon, 6),

"severity": round(zone_sev, 1),

"n_detections": len(items),

"dominant_class": Counter(diseased).most_common(1)[0][0]

if diseased else "Healthy",

"low_conf_share": sum(d["low_confidence"] for d in items) / len(items),

"poor_gps_share": sum(d["gps_accuracy_m"] > GPS_ACCURACY_LIMIT_M

for d in items) / len(items),

"bounds_m": {"x_min": col * CELL_SIZE_M, "x_max": (col + 1) * CELL_SIZE_M,

"y_min": row * CELL_SIZE_M, "y_max": (row + 1) * CELL_SIZE_M},

})

return zones

def decide_actions(zones):

"""A3. Resource optimization: spray ONLY zones above the threshold,

worst first, and never more than MAX_SPRAY_FRACTION of the field.

Dose is scaled to severity so mild zones get less chemical."""

budget = max(1, math.floor(MAX_SPRAY_FRACTION * len(zones)))

candidates = sorted([z for z in zones if z["severity"] >= SPRAY_THRESHOLD],

key=lambda z: z["severity"], reverse=True)

funded = {z["zone_id"] for z in candidates[:budget]}

for z in zones:

s = z["severity"]

if z["zone_id"] in funded:

high = s >= HIGH_THRESHOLD

z["recommended_action"] = "SPRAY - HIGH PRIORITY" if high else "SPRAY - STANDARD"

z["recommended_dose_pct"] = DOSE_HIGH_PCT if high else DOSE_STANDARD_PCT

elif s >= SPRAY_THRESHOLD:

z["recommended_action"] = "SPRAY DEFERRED (over budget) - re-scan"

z["recommended_dose_pct"] = 0

elif s >= MONITOR_THRESHOLD:

z["recommended_action"] = "MONITOR - re-scan in 2-3 days"

z["recommended_dose_pct"] = 0

else:

z["recommended_action"] = "NO ACTION"

z["recommended_dose_pct"] = 0

return zones

def flag_for_verification(zones):

"""A4. Risk register: 'incorrect spraying map'. Every zone is checked for

reasons the map might be wrong. Zones with reasons need extra scrutiny.

NOTE: even zones with NO reasons still need farmer approval (see gate)."""

for z in zones:

reasons = []

if z["poor_gps_share"] > POOR_GPS_ZONE_SHARE:

reasons.append(f"{z['poor_gps_share']:.0%} of GPS fixes worse than "

f"{GPS_ACCURACY_LIMIT_M} m: confirm zone position/boundary")

if z["n_detections"] < MIN_DETECTIONS_PER_ZONE:

reasons.append(f"Only {z['n_detections']} image(s): too little evidence")

if z["low_conf_share"] > LOW_CONF_ZONE_SHARE:

reasons.append(f"{z['low_conf_share']:.0%} of results below "

f"{CONFIDENCE_THRESHOLD:.0f}% confidence: review images")

z["needs_verification"] = bool(reasons)

z["verification_reasons"] = reasons

z["status"] = "AWAITING_FARMER_REVIEW"     # nothing is ever auto-approved

return zones

def release_for_spraying(zones, farmer_decisions):

"""A4 gate: the ONLY way a zone becomes part of a spray plan.

farmer_decisions = {zone_id: "approve" | "reject"}.

A zone with a spray action but no explicit 'approve' is NOT released."""

plan = []

for z in zones:

if not z["recommended_action"].startswith("SPRAY -"):

continue

if farmer_decisions.get(z["zone_id"]) == "approve":

z["status"] = "APPROVED_BY_FARMER"

plan.append(z)

else:

z["status"] = "HELD_NOT_APPROVED"

return plan

# =============================================================================

# 4. PIPELINE

# =============================================================================

def run_pipeline():

with tempfile.TemporaryDirectory() as folder:

batch, sim_truth, origin = simulate_batch(folder)     # [PLACEHOLDER]

classifier = load_classifier(sim_truth)

detections = []

for item in batch:                                    # [REAL] loop

try:

label, conf = classifier.predict(item["image_path"])

except ValueError as err:                          # corrupt image

print(f"[skip] {err}")

continue

detections.append({

**item,

"label": label,

"confidence_pct": round(conf, 1),

"low_confidence": conf < CONFIDENCE_THRESHOLD,

"severity": severity_score(label, conf),

})

zones = aggregate_zones(detections, origin)

zones = decide_actions(zones)

zones = flag_for_verification(zones)

return detections, zones

def summarise(zones, plan):

sprayed = [z for z in zones if z["recommended_action"].startswith("SPRAY -")]

saving = 100 * (1 - len(sprayed) / len(zones)) if zones else 0

return {

"zones_total": len(zones),

"zones_recommended_for_spray": len(sprayed),

"zones_needing_extra_verification": sum(z["needs_verification"] for z in zones),

"zones_released_after_farmer_approval": len(plan),

"estimated_chemical_saving_vs_blanket_spray_pct": round(saving, 1),

}

if __name__ == "__main__":

detections, zones = run_pipeline()

# ---- Final structured spraying map (what the dashboard would show) ----

spray_map = [{

"zone_id": z["zone_id"],

"severity": z["severity"],

"recommended_action": z["recommended_action"],

"recommended_dose_pct": z["recommended_dose_pct"],

"needs_verification": z["needs_verification"],

"verification_reasons": z["verification_reasons"],

"dominant_class": z["dominant_class"],

"n_detections": z["n_detections"],

"centre_latitude": z["centre_latitude"],

"centre_longitude": z["centre_longitude"],

"bounds_m": z["bounds_m"],

"status": z["status"],

} for z in zones]

# ---- [PLACEHOLDER] simulated farmer: approves every spray zone that has

# no verification reasons and rejects the rest. In the real system this

# is the farmer clicking approve/reject on the dashboard map.

decisions = {z["zone_id"]: ("reject" if z["needs_verification"] else "approve")

for z in zones}

plan = release_for_spraying(zones, decisions)

result = {"spraying_map": spray_map, "summary": summarise(zones, plan)}

# refresh status values after the gate ran

for entry, z in zip(result["spraying_map"], zones):

entry["status"] = z["status"]

print(json.dumps(result, indent=2))

with open("spraying_map.json", "w") as f:

json.dump(result, f, indent=2)

print("\nSaved -> spraying_map.json")